- Get Cv or Kv value from valve nameplate

Cv, Kv can be interchanged using these relationships

- Cv = 1.16*Kv

- Kv = 0.862*Cv

- Create a following table representing Equal Percentage Valve characteristics

Initial starting estimate is arbitrarily chosen; it will be calculated latter

An equal percentage characteristic is when the valve Cv increases by a constant percentage over a given travel value. For example, the Cv may increase by 5% for every 10% increase in valve travel.

Formulas used

- Use ‘Goal Seek’ to vary initial estimate to reach Cv at 100% Opening matching nameplate

Goal Seek animated file: